	
	Spartan Chemical Company, Inc.
Material Safety Data Sheet


[image: image1.png]


[image: image2.png]


SECTION I: PRODUCT INFORMATION
[image: image3.png]


SECTION I: PRODUCT INFORMATION
	Product Name or Number (as it appears on label):
	Product Division:

	CONSUME DROP-IN-A-DRAIN
Product Number: 3205
	Janitorial


	Spartan Chemical Company, Inc.
	Product/Technical Information:
	1-(800)-537-8990

	1110 Spartan Drive
	Medical Emergency:
	1-(888)-314-6171 (24 hours)

	Maumee OH 43537
	Chemical Leak/Spil Emergency:
	CHEMTREC 1-(800) 424-9300 (24 hours)


	Shipping Description:
	Cleaning compounds, powder, n.o.s.


	NFPA Ratings:
	[image: image4.png]



	HMIS Ratings:
	[image: image5.png]




	Health:
	1- Slight
	Health:
	1- Slight

	Fire:
	0 - Minimal
	Fire:
	0 - Minimal

	Reactivity:
	0 - Minimal
	Reactivity:
	0 - Minimal

	[image: image6.png]



	[image: image7.png]



	Pers. Prot. Equip.:
	See Section VIII


[image: image8.png]


[image: image9.png]


SECTION II: HAZARDOUS INGREDIENTS
[image: image10.png]


SECTION II: HAZARDOUS INGREDIENTS
(Listed when present at 1% or greater, carcinogens at 0.1% or greater) All component chemicals are listed or exempted from listing on the "TSCA Inventory" of chemical substances maintained by the U.S. Environmental Protection Agency. 

	Chemical Name(s)
	% W 
	CAS Registry No.
	Table Z- 1 - A
	NTP, IARC or OSHA Carcinogen 

	
	
	
	TWA mg/m3 
	STEL mg/m 3
	CELING mg/m3 
	

	Viable bacterial cultures 
	NA
	N/A
	Not Established 
	Not Established
	Not Established
	No 


[image: image11.png]


[image: image12.png]


SECTION III: PHYSICAL DATA
[image: image13.png]


SECTION III: PHYSICAL DATA 

	Boiling Point:
	NA °F
	Vapor Pressure:
	NA 

	Vapor Density (AIR = 1):
	Unknown
	Solubility in Water:
	Minimal

	pH:
	1 % 6.5 - 8.5
	Specific Gravity (H2O=1):
	Bulk Density 0.66 - 0.77 gm/cm3

	Evaporation Rate (but.ace.=1):
	<1
	Percent Solid by Weight:
	100

	Physical State:
	Powder
	[image: image14.png]



	[image: image15.png]





	Appearance & Odor:
	Tan powder in 1 oz. water soluble packets 


[image: image16.png]


[image: image17.png]


SECTION IV: FIRE & EXPLOSIVE HAZARD DATA
[image: image18.png]


SECTION IV: FIRE & EXPLOSIVE HAZARD DATA
	Flash Point:
	None
	Method Used:
	ASTM-D56

	Flammable Limits:
	N/A
	Flame Extension:
	N/A


	Extinguishing Media:
	Product does not support combustion. Use extinguishing media appropriate for surrounding fire.

	Special Fire Fighting Procedures:
	Wear NIOSH approved self-contained breathing apparatus and protective clothing. Cool fire-exposed containers with water spray.

	Unusual Fire & Explosive Hazards:
	Combustion products are toxic. 


[image: image19.png]


[image: image20.png]


SECTION V: HEALTH HAZARD DATA
[image: image21.png]


SECTION V: HEALTH HAZARD DATA 

	Threshold Limit Value:
	Not established
	Primary Routes of Entry:
	Inhalation, Skin Contact, Eyes & Oral


	Effects of Overexposure-
Conditions to Avoid:
	May cause eye and skin irritation with symptoms such as pain, redness and swelling of the conjunctiva.
Harmful if swallowed with symptoms such as pain, nausea, vomiting and diarrhea.
Inhalation of product dust may cause respiratory irritation.
This product contains living bacterial cultures. The bacteria in this product may cause infection if introduced into an open wound, broken skin or mucous membrane in sufficient quantity. 
Do not get in eyes, on skin, or on clothing. Do not swallow. Avoid breathing product dust. Avoid contact with open wounds, broken skin or mucous membranes. Wash thoroughly after handling. 

	Conditions Aggravated by Use:
	Use of this product may aggravate preexisting skin; eye and respiratory disorders including asthma and dermatitis.


Emergency & First Aid Procedures: 

	Eyes:
	Flush eyes with water for at least 15 minutes. Remove contact lenses. Get medical attention. 

	Skin:
	Wash skin with plenty of soap and water. Apply topical antiseptic agent to open wounds or broken skin. Get medical attention if irritation persists

	Ingestion:
	Do not induce vomiting. Drink one or two glasses of water to dilute product. Get medical attention. Do not give anything by mouth to an unconscious person.

	Inhalation:
	Move person to fresh air. Get medial attention if irritation persists.


[image: image22.png]


[image: image23.png]


SECTION VI: REACTIVITY DATA
[image: image24.png]


SECTION VI: REACTIVITY DATA
	Stability:
	Stable
	Incompatible Materials:
	Strong oxidizers; strong acids or alkali compounds may inactivate biological cultures.

	Hazardous Decomposition Products:
	CO, CO2
	Hazardous Polymerization:
	Will Not Occur


[image: image25.png]


[image: image26.png]


SECTION VII: SPILL OR LEAK PROCEDURES
[image: image27.png]


SECTION VII: SPILL OR LEAK PROCEDURES 

	Steps to be Taken in Case 
Material is Released or Spilled:
	Small quantities may be flushed into sanitary sewer. Larger quantities should be picked up and transferred to suitable containers for disposal.

	Waste Disposal Method:
	Dispose of in compliance with all federal, state and local laws and regulations.


[image: image28.png]


[image: image29.png]


SECTION VIII: SPECIAL PROTECTION INFORMATION
[image: image30.png]


SECTION VIII: SPECIAL PROTECTION INFORMATION
	Respiratory Protection:
	Not normally required when good general ventilation is provided.

	Ventilation:
	Provide good general ventilation. Local exhaust ventilation may be necessary for some operations.

	Protective Gloves(Specify Type):
	Rubber or other impervious gloves

	Eye Protection(Specify Type):
	Not required.

	Other Protective Equipment:
	See 29 CFR 1910.132-138 for further guidance.


[image: image31.png]


[image: image32.png]


SECTION IX: SPECIAL PRECAUTIONS
[image: image33.png]


SECTION IX: SPECIAL PRECAUTIONS
	Precautions; Handling & Storing:
	Store in cool; dry location. Keep container tightly sealed when not in use. Do not contaminate food or feed.

	Other Precautions:
	Wash thoroughly after handling. Keep out of reach of children.


	[image: image34.png]


© SCC 11/01/2007
	Name:
	Ronald T. Cook
	Title:
	Manager, Regulatory Affairs

	CONSUME DROP-IN-A-DRAIN
	Effective Date:
	11/01/2007
	Supercedes:
	11/15/2004 

	Ref: 29 CFR 1910.1200 (OSHA)
	Changes:
	General update


	This document has been prepared using data from sources considered technically reliable. It does not constitute a warranty, express or implied, as to the accuracy of the information contained herein. Actual conditions of use and handling are beyond sellers control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State and Local laws and regulations.


